| MONTHLY PROGRESS REPORT |  |
| --- | --- |
| Contractor Name: University of Notre Dame (Michael Lemmon) |  |
| Contractor Address: Office of Research, 940 Grace Hall, Notre Dame, IN 46556 |  |
| Contract/Purchase Order No. W9132T-10-C-0008 (prime contract no.) | Task Order No. |
| Project Title: Design and Simulation of Intelligent Control Architecture for Military Microgrids |  |
| Period Covered: April 1 2011 – May 1, 2011 |  |
| POC/COR (Reference Paragraph 5 of the SOW): |  |
| Achievements (Describe by task. Add additional tasks, if needed.): task numbers refer to tasks in Odyssian’s original contract |  |
| Task II: Model and Simulate Intelligent Microgrid Extended single-phase simulation of Odyssian testbed to consist of 3 200 W microsources and 4 e-boards handling 120 W loads. A preliminary dispatch logic was developed and tested on this simulation. |  |
| Task III: Distributed Control Algorithm Development Began development of dispatch logic for Odyssian single phase testbed. |  |
| Task VI: Develop Wireless Communication No activity |  |
| Task VII: Develop Wireless Distributed Control No activity |  |
| Problems Encountered (Describe by task. Add additional tasks, if needed): |  |
| Task II: None |  |
| Task III: None |  |
| Task VI: None |  |
| Task VII: None |  |
| Open Items (List items that require action by the Contractor or the Government):s No open items |  |
| Summary Assessment and Forecast (Provide an overall assessment of the work and a forecast of contract completion): The simulation of Odyssian’s single phase testbed was extended to include 3 microsources (200 W) with 4 eboards (120 W). The simulation models were developed from last month’s microsource models which modeled the inverter as a controlled voltage source. A block diagram for the simulated single phase testbed is shown to the right. The total amount of microgrid genertion is 600 W with a total load of 480 W. Note that on the point of common coupling, the microsources and each e-board, we’ve attached a “dispatch” agent. These agents are computational subsystems that measure power and frequency, transmit those measurements to other agents in the grid, whose then recomputed the real power setpoints of the microsources. The dispatch algorithms for the Odyssian testbed are simpler than the earlier dispatch agents. We don’t assume event-triggered signaling, as this adds an extra layer of complexity to the hardware programming. Moreover, the need for highly distributed computation is not necessary since all sources and all loads are interconnected. These differences between the Odyssian testbed and the original mesh microgrid being tested at UW make the implementation of both the dispatch and load shedding logic much simpler. This dispatch logic developed for the Odyssian demonstration consists of three types of computational agents. The first agent is attached to the point of common coupling (PCC) where the microgrid connects to the main power grid. This agent simply monitors whether real power is flowing through the PCC and signals to other agents whether the microgrid has islanded or not. The second type of agent is attached to the microsources. These agents monitor real power and frequency at the source’s terminals. These agents determine whether or not the microsource is connected and adjust the requested real power (Preq). The third type of agent is attached to the eboards. These agents receive information regarding the amount of excess generation capacity in the microgrid. They use this information to determine a frequency trip at which non-critical loads are reconnected to the grid. |  |